K.Amrutha                                                           ITA0515

192425366                                                 COMPUTER VISION

1. Object Boundary Detection

(a) Interpretation:
Boundary detection identifies the edges between different tissues in a medical image, helping doctors accurately identify organs or abnormal regions.

(b) Key Challenges:

- Image noise

- Low contrast

- Blurred edges

- Similar tissue intensities

(c) Suitable Edge Detection Method:
Canny Edge Detection

(d) Justification:
Canny removes noise using Gaussian filtering, detects strong and weak edges, and provides accurate, continuous boundaries.

(e) Conclusion:
Canny edge detection is the most suitable method because it provides precise and reliable tissue boundaries.

2. Simple Segmentation

(a) Interpretation:
The object is clearly different from the background based on intensity, making segmentation easy.

(b) Key Features:

- High intensity difference

- Clear object boundaries

- Uniform background

(c) Suitable Technique:
Threshold Segmentation (Otsu's Thresholding)

(d) Justification:
Thresholding separates foreground and background automatically using intensity values.

(e) Conclusion:
Threshold segmentation is simple, fast, and effective for images with clear intensity differences.

3. Complex Segmentation

(a) Interpretation:
Objects have overlapping intensity values, making separation difficult.

(b) Key Issues:

- Intensity overlap

- Noise

- Touching objects

- Irregular shapes

(c) Suitable Method:
Watershed Segmentation

(d) Justification:
Watershed uses gradient information to separate connected or overlapping regions effectively.

(e) Conclusion:
Watershed segmentation accurately handles complex images with overlapping objects.

4. Broken Edges

(a) Interpretation:
Detected edges are discontinuous due to weak gradients or noise.

(b) Key Issues:

- Noise

- Weak edges

- Poor image quality

- Low contrast

(c) Suitable Method:
Morphological Closing (Dilation + Erosion)

(d) Justification:
Morphological closing connects broken edges and improves boundary continuity.

(e) Conclusion:
Morphological operations produce continuous and connected object boundaries.

5. Combined Requirement

(a) Interpretation:
The system must remove noise while preserving important edges for defect detection.

(b) Key Issues:

- Image noise

- Edge preservation

- Small defect detection

- Accuracy

(c) Sequence of Techniques:

- Median Filter (Noise Removal)

- Canny Edge Detection (Edge Extraction)

(d) Justification:
The median filter removes noise without blurring edges, and Canny accurately detects important boundaries.

(e) Conclusion:
Using a Median Filter followed by Canny Edge Detection provides effective noise removal while preserving important edges for industrial inspection.